National Higher School of Autonomous Systems Technology

Electronics Final Exam/ Tuesday, June 24th, 2025

Duration: 1h30

Exercise 1:

Consider the circuit below. Assume the diode is ideal, and the input voltage is a sine wave with amplitude of 20V and a frequency  f=100 Hz . With  RL=10 kΩ .

- Draw approximately the output voltage Vout for the first period, and precise the time and voltage values knowing that =5 kΩ .

We replace the resistor R with a capacitor =1 μF :

- Draw approximately the output voltage Vout for the first period.

- Indicate below your graph the intervals when the capacitor is charging and discharging.

For the voltage source Vin with an amplitude of 15V and a period T=20ms, and for the same output load RL :

Draw the circuit that gives the following output voltage. Precise the value of each element (1 point)

Exercise 2

Consider the two-port network (quadripole) shown in the figure below, constructed with two coupled voltage generators and four identical resistors of value R.

- Express the impedance matrix [Z] in terms of R,

- Draw the equivalent circuit that represents this impedance matrix. Show all impedance elements clearly.

- For the two-port network loaded with a resistance RL at the output port, express in terms of E, R, and RL:

- Voltage gain (Av)

- Input impedance (Zin)

- Output impedance (Zout)

Problem:

Two electronics designers created two different circuits (A and B) to solve the same problem. The circuits (A) and (B) are depicted in the Figure 4 and 5. The designers Fixed the Quiescent point (Q-point) in the middle of the load line. (βDC=213 for the 2N2222 and βDC=141 for the 2N3904 )

Part1: DESIGN A

- Knowing that: RC=1.65kΩ and R2=2.7kΩ and the saturation current is: 2.34mA, calculate the values of R1, and RE. Deduce the value of IE.

- What is the name of this biasing circuit?

- What is the purpose of this circuit?

- How do we call capacitors C1, C2? How do we call capacitor C3?

- Give the ac-equivalent circuit. (Recall that re=25mVIE).

- Calculate the ac-voltage gain (GV).

- Give the input impedance (Zin) and output impedance (Zout) of the circuit.

Figure 4: Design A

Figure 5: Design B

Part2: Comparison between designs A and B

Note that the Q-point for the Design B is the same as for the Design A

- What is the name of the biasing circuit in Design B?

- Transistor 2N2222 burned and the technician found in his possession a 2N3904. He replaced it in the circuit.

2. a what happens if the original circuit was the one of Design A

2. b what happens if the original circuit was the one of Design B

- Which is the best design? Justify.