National Higher School of Autonomous Systems Technology

Electronics Mid-Term/ Sunday, June 15th, 2025

Duration: 1h30

Exercise 1:

Given the quadripole in the following figure:

- Calculate the impedance matrix Z.

- Calculate the hybrid matrix H.

- Using the hybrid parameters, give the equivalent circuit of the quadripole.

A load RL is connected to the output of the quadripole:

- Determine the voltage gain Av=V2V1 .

Exercise 2

1. Consider a quadripole (two-port network) Q defined by its hybrid matrix H=h11h12h21h22. Determine its impedance matrix Z as a function of the parameters h₁₁, h₁₂, h₂₁, and h₂₂. Specify the conditions for the existence of this transformation.

2. Consider a quadripole Q defined by its impedance matrix Z=z11z12z21z22.. Determine its hybrid matrix H as a function of the parameters z₁₁, z₁₂, z₂₁, and z₂₂. Specify the conditions for the existence of this transformation.

3. Consider two quadripoles Q₁ and Q₂ defined respectively by their hybrid matrices:

H1=1234 and H2=1124.

We form quadripole Qt by the series-series connection of the two quadripoles Q₁ and Q₂.

a) Draw the schematic diagram showing the series-series connection of the two quadripoles Q₁ and Q₂.

b) Find the equivalent hybrid matrix of quadripole Qt.

Exercise 3: (5pts):

Given the quadripole in the following figure, answer the questions:

- True or false, this configuration is known as the π-Form quadripole

- Which of the following stands for symmetrical quadripole:

a) Z1=Z2=Z3;       b) Z1=Z3;      c) Z2=Z3;       d) Z1=Z2;

- Let Z1=Z2=R and Z3 a capacitor. What is the of role of this circuit? Justify with simple reasoning.

- Which matrix is better to study this circuit:

a) Z-matrix;         b) Y-matrix;      c) H-matrix;         d) G-matrix;        Justify

- Let Z1, Z2 and Z3 resistors. The quadripole issymmetrical ended with RL, a load. By carefully choosing their values we can:

a) get an amplification of the input voltage; b) get an attenuation of the input voltage

c) get impedance matching; d)  inverse the polarity;   Justify with quick calculations.