Exercise1:

Given the circuit below. The first voltage source V1 is a sine wave with an amplitude of 2V and a frequency f=100 Hz. The second voltage source V2 is a sine wave with an amplitude of 1V and a frequency f=100 Hz.

- Draw the output voltage Vout for the first period, and precise the time and voltage values. D1 and D2 are ideal diodes. (1 point)

- Draw the output voltage Vout if the second voltage source V2 has a phase of 180 degrees. D1 and D2 are real diodes with a voltage drop of 0.7V . (1 point)

- Replace the resistor R2 with a capacitor C=1 μF. Draw approximately the output voltage Vout for the first period. (Consider the original V2 without the phase. D1 and D2 are ideal diodes.) (2 points)

- Explain why is the output voltage Vout in the second period the same as or different than the first period. (0.5 point)

The output voltage in the second period is different because the capacitor does not discharge.