National Higher School of Autonomous Systems Technology

Electronics Mid-Term/ Sunday, June 15th, 2025

Duration: 1h30

Exercise 3: (5pts):  (0.5, 2*0.5, 2*0.5, 2*0.5, 3*0.5)

Given the quadripole in the following figure, answer the questions:

- True or false, this configuration is known as the π-Form quadripole

- Which of the following stands for symmetrical quadripole:

a) Z1=Z2=Z3;       b) Z1=Z3;      c) Z2=Z3;       d) Z1=Z2;

- Let Z1=Z2=R and Z3 a capacitor. What is the of role of this circuit?  Low Pass Filter

Justify: Z3=1/jc𝟂 goes to infinity for small frequencies (𝟂     0) and it becomes open circuit which means the output equals the input minus voltage drop caused by resistances. Z3=1/jc𝟂 goes to zero for high frequencies (𝟂     ∞) and it becomes short circuit which means the output equals 0.

- Which matrix is better to study this circuit:

a) Z-matrix;         b) Y-matrix;      c) H-matrix;         d) G-matrix;        Justify: it consists of impedances.

- Let Z1, Z2 and Z3 resistors. The quadripole is symmetrical ended with RL, a load. By carefully choosing their values we can:

a) get an amplification of the input voltage; b) get an attenuation of the input voltage

c) get impedance matching; d)  inverse the polarity;   Justify: give the voltage gain in function of resistances and show that it is less than 1 and that it is positive.